# AI-Based Three-Phase Voltage Unbalance Detection and Severity Classification

Internship Capstone Project Report

Prepared as an educational prototype using Python, scikit-learn and Streamlit.

# 1. Abstract

Three-phase voltage unbalance can contribute to additional heating, reduced efficiency and abnormal operation of three-phase equipment. This project presents a software prototype that receives phase R, Y and B RMS voltage readings, validates the data, calculates average voltage and a voltage-unbalance percentage, and predicts a severity class using machine-learning classifiers. Decision Tree and Random Forest models are trained and evaluated. A Streamlit dashboard allows a user to enter new readings and inspect predictions. The included dataset is synthetic and supports reproducible demonstration only; it is not evidence of field performance.

# 2. Problem Statement

Develop an AI-assisted system to identify balanced, mildly unbalanced and severely unbalanced conditions from three-phase voltage measurements, evaluate model performance and provide a simple dashboard for new readings.

# 3. Objectives

Read and validate three phase-voltage values.

Calculate average voltage and voltage-unbalance percentage.

Create severity labels using documented illustrative thresholds.

Train and compare Decision Tree and Random Forest classifiers.

Evaluate using a held-out test set and a separate unseen dataset.

Display voltage values, calculated unbalance and AI prediction in a dashboard.

# 4. System Architecture

Voltage dataset / simulated readings → data cleaning → feature engineering → model training and evaluation → saved model → Streamlit dashboard → severity prediction.

# 5. Dataset and Data Validation

The supplied training dataset contains 2400 synthetic rows and three phase voltage inputs. A separate file contains 300 unseen synthetic readings for demonstration. Missing phase readings are excluded during training; the model pipeline includes median imputation for feature values. All reported outcomes must be described as synthetic-data results.

Input columns: Vr_V, Vy_V and Vb_V. Derived columns: Average_V, Unbalance_Percent and Severity.

# 6. Electrical Feature Calculation

Average phase voltage: V_avg = (V_R + V_Y + V_B) / 3

Chosen voltage-unbalance metric: VU(%) = [max(|V_R − V_avg|, |V_Y − V_avg|, |V_B − V_avg|) / |V_avg|] × 100

This is one possible engineering metric. Some standards use the ratio of negative-sequence to positive-sequence voltage. The correct definition must be confirmed with the project supervisor and relevant standard before deployment.

# 7. Severity Labels

| Class | Illustrative threshold |
| --- | --- |
| Balanced | VU ≤ 1.0% |
| Mildly Unbalanced | 1.0% < VU ≤ 2.0% |
| Severely Unbalanced | VU > 2.0% |

These thresholds are configurable demonstration labels, not universal compliance limits.

# 8. Machine-Learning Methodology

The dataset is split into 80% training and 20% testing with stratification and a fixed random seed. Two classifiers are trained: a Decision Tree with depth control and a Random Forest with 200 trees. A scikit-learn pipeline applies median imputation before classification. Weighted precision, recall and F1-score are calculated along with accuracy and confusion matrices. The selected model is the model with the highest weighted F1-score on the held-out test set.

# 9. Dashboard

The Streamlit dashboard includes three tabs: live input prediction, unseen test data, and project notes. Users enter three phase voltages and see average voltage, unbalance percentage, AI severity, a threshold-based reference class and a bar chart. The separate reference class helps expose disagreements instead of hiding them.

# 10. Running the Project

Install Python 3.10 or newer.

Open a terminal in the extracted project folder.

Run: pip install -r requirements.txt

Run: python train_model.py

Run: streamlit run app.py

Open the local Streamlit URL shown in the terminal and test values such as 230, 230, 230 V and 230, 225, 218 V.

# 11. Testing and Results

Run train_model.py to generate the actual metrics for this environment. The script writes outputs/metrics.json, confusion-matrix images, the saved best model and predictions for unseen_test_data.csv. Insert the values from metrics.json into the final presentation/report after running the code. Do not claim a performance figure without executing the training script.

# 12. Limitations and Safety

Training and unseen datasets are synthetic, not measured field data.

The severity labels inherit the chosen illustrative thresholds; model scores do not validate those thresholds.

The selected metric may differ from a standard or utility definition.

A software dashboard is not a certified protection or safety system.

For real voltage measurement, use rated isolated transducers/meters and qualified supervision. Never connect Arduino or a PC directly to mains voltage.

Validate against calibrated measurement equipment and a representative field dataset before any operational use.

# 13. Future Enhancements

Collect a labelled real-world dataset; implement sequence-component unbalance if required; add time-series trend and alarm logging; test drift and sensor faults; add role-based access and secure storage; deploy only after electrical safety and standard compliance review.

# 14. Conclusion

The prototype demonstrates the end-to-end workflow of voltage data processing, engineering feature calculation, supervised classification, evaluation and dashboard visualization. Its main value is as a reproducible learning and internship demonstration. Real-world conclusions require field data, a validated engineering definition and approved measurement hardware.

# Appendix: Project files

train_model.py — training and evaluation; app.py — Streamlit dashboard; data/voltage_unbalance_dataset.csv — synthetic training data; data/unseen_test_data.csv — synthetic unseen readings; requirements.txt — dependencies.

# 15. Executed Baseline Results

The provided training script was executed successfully on the supplied synthetic dataset. Both the Decision Tree and Random Forest achieved 1.0000 accuracy, weighted precision, weighted recall and weighted F1-score on the held-out test split. The selected model was Decision Tree (tie resolved by model order). On the separate 300-row synthetic unseen file, the model also achieved 1.0000 accuracy.

Interpretation warning: these perfect scores are expected because the target severity classes are generated directly from the same voltage-unbalance percentage included as a model feature. This is a controlled proof-of-work demonstration, not evidence that the model will achieve perfect performance on real measurements. For a stronger real-world evaluation, collect independently labelled measured data, verify labels against the applicable engineering standard, and evaluate on a separate site/time-based dataset.